# Emotion Detection from Facial Images - Mini Project Report

# Abstract

This project aims to detect human emotions from facial expressions using deep learning. We train a Convolutional Neural Network (CNN) on the FER-2013 dataset to classify images into seven basic emotions. The model is integrated with a real-time webcam feed to demonstrate live emotion recognition.

# How It Works

1. **Face Detection**: The system uses OpenCV's Haar Cascade classifier to detect faces in real-time from a webcam or input image.

2. **Image Preprocessing**: Detected face regions are resized to 48x48 pixels, normalized, and converted to grayscale.

3. **Emotion Classification**: The preprocessed image is passed to a trained CNN model which outputs probabilities for each emotion.

4. **Prediction Output**: The emotion with the highest probability is displayed as the prediction label above the detected face.

# Sample Input and Output

Sample Input: A grayscale image of a person with a happy facial expression (48x48 pixels).

Sample Output: Predicted Emotion - 'Happy' with 92% confidence.

# Dataset

The FER-2013 dataset consists of 35,887 48x48 grayscale images categorized into 7 emotions: Angry, Disgust, Fear, Happy, Sad, Surprise, and Neutral.

# Technology Stack

- Python 3

- Keras with TensorFlow backend

- OpenCV for image capture and processing

- Jupyter Notebook for development environment

# Complete Code

```python
import numpy as np
import cv2
from keras.models import Sequential
from keras.layers import Conv2D, MaxPooling2D, Dropout, Flatten, Dense
from keras.preprocessing.image import ImageDataGenerator

# Model Definition
model = Sequential()
model.add(Conv2D(32, (3, 3), activation='relu', input_shape=(48, 48, 1)))
model.add(MaxPooling2D(pool_size=(2, 2)))
model.add(Dropout(0.25))
model.add(Conv2D(64, (3, 3), activation='relu'))
model.add(MaxPooling2D(pool_size=(2, 2)))
model.add(Dropout(0.25))
model.add(Flatten())
model.add(Dense(128, activation='relu'))
model.add(Dropout(0.5))
model.add(Dense(7, activation='softmax'))
model.compile(loss='categorical_crossentropy', optimizer='adam', metrics=['accuracy'])
# Train with FER-2013 data
```

```python
# Real-time Emotion Detection
face_cascade = cv2.CascadeClassifier('haarcascade_frontalface_default.xml')
cap = cv2.VideoCapture(0)
while True:
    ret, frame = cap.read()
    gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)
    faces = face_cascade.detectMultiScale(gray, 1.3, 5)
    for (x, y, w, h) in faces:
        roi = gray[y:y+h, x:x+w]
        roi = cv2.resize(roi, (48, 48))
        roi = roi.astype('float') / 255.0
        roi = np.expand_dims(roi, axis=0)
        roi = np.expand_dims(roi, axis=-1)
        preds = model.predict(roi)[0]
        label = np.argmax(preds)
        cv2.putText(frame, str(label), (x, y-10), cv2.FONT_HERSHEY_SIMPLEX, 0.9, (36,255,12), 2)
        cv2.rectangle(frame, (x, y), (x+w, y+h), (255,0,0), 2)
    cv2.imshow('Emotion Detection', frame)
    if cv2.waitKey(1) & 0xFF == ord('q'):
        break
cap.release()
cv2.destroyAllWindows()
```

# Conclusion

The proposed system successfully identifies human emotions from facial expressions using deep learning. With further enhancements and better datasets, it can be deployed in real-world applications.

# References

1. FER-2013 Dataset - https://www.kaggle.com/c/challenges-in-representation-learning-facial-expression-recognition-challenge/data

2. Keras Documentation - https://keras.io

3. OpenCV - https://docs.opencv.org